TECHNICAL STANDARDS & REGULATORY BLUEPRINT

Indian Standards (BIS) & Regulatory Specification for Electric Immersion Water Heaters

Target Product: Electric Immersion Water Heater (Model EWH-1500, 1500W, 230V AC) | Zyntrix Ref: ZYN-BIS-EWH-2026

| Product Category: Electrical Appliances | Compulsory Scheme: Scheme-I (ISI Mark) | Primary Standard: IS 302-2-201:2008 | Statutory Mandate: DPIIT QCO S.O. 189(E) |
| --- | --- | --- | --- |

# 1. Knowledge Base Analysis & Regulatory Distinction

During automated and empirical verification of the repository knowledge base against the sample demonstration laboratory report (Electric_Immersion_Water_Heater_Lab_Report.pdf), an essential regulatory finding was uncovered:

Current Knowledge Base State: Zyntrix currently indexes IS 302-2-15:2009 in its verified dataset (data/bis/verified/IS_302_2_15_2009). However, IS 302-2-15 specifically regulates liquid-heating appliances such as electric kettles, coffee makers, and boiling urns. It is technically distinct from portable immersion heating rods.

Correct Primary Standard: Portable immersion rods are governed by IS 302 (Part 2/Sec 201) : 2008 ('Safety of Household and Similar Electrical Appliances - Part 2: Particular Requirements - Section 201: Electric Immersion Water Heaters'), evaluated in conjunction with IS 302-1:2008.

Historical Predecessor: IS 302-2-201:2008 formally superseded IS 368:1992 ('Electric Immersion Water Heaters - Specification'). Modern manufacturing licenses issued by the Bureau of Indian Standards (BIS) reference IS 302 (Part 2/Sec 201).

Compulsory Statutory Mandate: Electric immersion water heaters fall under the Electrical Appliances (Quality Control) Order, 2003 (S.O. 189(E)) enacted by DPIIT, Ministry of Commerce and Industry. Compliance with Scheme-I (ISI Mark) is mandatory by law under the BIS Act, 2016.

# 2. Official Indian Standards (BIS) Master Matrix

| Standard Designation | Title / Specification | Technical Scope & Purpose | Legal Mandate |
| --- | --- | --- | --- |
| IS 302 (Part 2/Sec 201) : 2008 | Particular Requirements for Electric Immersion Water Heaters | Submersion dielectric limits, liquid depth markings, dry-boil safety, handle insulation. | MANDATORY (Scheme-I ISI Mark, QCO S.O. 189(E)) |
| IS 302-1 : 2008 / 2024 | General Requirements for Household Electrical Appliances | General safety: wattage deviation (+5%/-10%), leakage current, earthing continuity, mechanical shock. | MANDATORY (Base electrical safety standard) |
| IS 1293 : 2019 | Plugs and Socket-Outlets for Household and Similar Purposes | Molded 3-pin 6A / 16A plug fitted to the power cord must bear independent ISI certification. | MANDATORY (Plugs & Sockets QCO) |
| IS 694 : 2010 | PVC Insulated Cables for Working Voltages up to 1100 V | Supply flexible cord (3-core, copper conductor, PVC sheath) must carry independent ISI marking. | MANDATORY (Cables QCO) |
| PM/IS 302 (Part 2/Sec 201) | BIS Product Manual for Immersion Water Heaters | Scheme of Inspection and Testing (SIT), factory routine tests, acceptance test lot sizes. | STATUTORY (BIS Licensing Manual) |
| IS 4905 : 2015 | Random Sampling and Randomization Procedures | Statistical framework for lot inspection, sample selection, and surveillance audits. | Statutory Sampling Standard |

# 3. Clause-by-Clause Compliance Mapping (Lab Report Evaluation)

Direct mapping of test results from Electric_Immersion_Water_Heater_Lab_Report.pdf (Model EWH-1500, 1500W, 230V AC) against statutory Indian Standard clauses and compliance limits:

| Lab Test Parameter | Lab Value | IS Clause | Acceptable Criterion / Tolerance | Verdict |
| --- | --- | --- | --- | --- |
| Rated Power Test | 1492 W | IS 302-1 Cl 10 | Permissible tolerance: +5% to -10% (1350 W - 1575 W) | PASS (-0.53%) |
| Input Voltage | 230 V AC | IS 302-1 Cl 6 | Nominal single-phase voltage (230 V ± 10%, 50 Hz) | PASS |
| Insulation Resistance | 25 MΩ | IS 302-1 Cl 13/16 | ≥ 2.0 MΩ (cold condition), ≥ 5.0 MΩ (after conditioning) | PASS (25 MΩ > 2 MΩ) |
| Electric Strength | No Breakdown | IS 302-1 Cl 13/16 | 1500 V AC sinusoidal applied for 60 seconds without breakdown | PASS |
| Leakage Current | 0.32 mA | IS 302-1 Cl 13 | Max allowable limit ≤ 0.75 mA for Class I portable appliances | PASS (0.32 ≤ 0.75 mA) |
| Earthing Continuity | 0.08 Ω | IS 302-1 Cl 27 | Resistance between earth pin and exposed metal parts ≤ 0.10 Ω | PASS (0.08 ≤ 0.10 Ω) |
| Temperature-Rise | Within limit | IS 302-1 Cl 11 | Non-metallic handle grip temperature rise ≤ 60 K | PASS |
| Mechanical Strength | No damage | IS 302-1 Cl 21 | Withstand 3 impacts of 0.5 J using spring hammer; 1m drop test | PASS |
| Marking & Labeling | Compliant | IS 302-1/2-201 Cl 7 | V, W, Model, Maker, Min/Max immersion level lines, ISI Mark + CM/L | PASS |
| Submersion Ingress | Compliant | IS 302-2-201 Cl 15 | Submersion to maximum water level for 24h + dielectric test | PASS |
| Abnormal Operation | No Hazard | IS 302-2-201 Cl 19 | Dry-boil operation without fire, explosion, or live conductor exposure | PASS |

# 4. Factory Quality Control & Scheme of Inspection and Testing (SIT)

Routine Tests (100% of Production): Every single immersion water heater manufactured must undergo: (1) High-voltage flash electric strength test at 1500 V AC for 1 second, (2) Earth continuity test proving resistance ≤ 0.10 Ω, and (3) Wattage consumption test at rated 230 V AC.

Acceptance Tests (Per Batch Lot): Representative statistical samples drawn per IS 4905 must undergo insulation resistance measurement, leakage current test (≤ 0.75 mA), cord anchorage test, and inspection of permanent markings.

Type Tests (Qualification & Annual Audit): Comprehensive evaluation of all 32 clauses performed at initial license grant and annual surveillance by BIS-accredited testing laboratories.

# 5. Integration Blueprint for Zyntrix Knowledge Base

To provide full, native BIS coverage in Zyntrix for immersion heaters, the platform has been updated with: (1) Verified standard directory data/bis/verified/IS_302_2_201_2008/ with full regulatory provenance, (2) Updated taxonomy rule APP-ELECTRICAL-002 explicitly matching 'immersion heater' to IS 302-2-201:2008, and (3) Clause-level test roadmaps for NABL laboratory testing.

STATUTORY REGULATORY ADVISORY: 
This specification is compiled in accordance with the Bureau of Indian Standards Act, 2016 and the Electrical Appliances (Quality Control) Order, 2003 (S.O. 189(E)). Generated by Zyntrix AI Compliance Compiler (SIH Problem Statement 26107).